Release Notes AP72W_1.0.58.10

1. Release Overview

| Product | WS-AP72W |
| --- | --- |
| Version | AP72W_1.0.58.10 |
| Release Date | 2026-09-18 |
| Build ID |  |
| Release Type | GA |
| Upgrade Recommendation | Recommended |

2. Supported Platforms

- All hardware versions of WS-AP72W are supported

3. Key Highlights

- Fixed known bugs and resolved the issue where the AP works abnormally under certain scenarios

- New Features

- None

5. Enhancements

- None

6. Bug Fixes

- Fixed the issue where the WiFi radio was disabled when the gateway was unreachable via ping

- Fixed the issue where, after the TV entered sleep mode, AP waking up the STA caused it to wake up.

- Fixed the issue where the AP failed to connect to the AC when the AP and AC were assigned to different subnets.

- Known Issues

- None.

- Compatibility & Dependencies

- Compatible AC versions

| Unify AC | UnifyAC_CR_1.0.6.3 or later |
| --- | --- |

9. Upgrade Instructions

- Upgrade from Unify AC at UnifyAC_CR_1.0.6.3 or later

Notes:
- Downtime: lower than 5 minutes (single)
- Config retained: Yes

10. Downgrade / Rollback

- Supported version: AP72W_1.0.44 or later

- Config compatibility: Full

- TAC Advisory

- Recommended upgrade scenarios: All deployments

- Known risks in previous versions:

- Deployment Notes

- Suitable for standard managed WiFi scenarios

13. PoE Power Budget

- Optical-electrical Hybrid Cable or DC Power Supply Scenario (with 100watts PSU)

Dual PSE Port can supports up to 30W Power out.

- PoE Power Supply Scenario

One PSE Port can supports up to 10W Power out, do not use two PSE ports simultaneously .

14. Thermal Advisory

- Passive Heat Dissipation, do not cover the surface with any objects

15. Security Updates

Not applicable

16. Documentation & Support

- Support: support@westalatech.com

17. Disclaimer

- Westala reserves the right to modify specifications without notice.