DPSK User

Configuration Guide

Unify Access Controller (UnifyAC)

| Applicable Version: | UnifyAC_CR_1.0.6.0 or later and AP 1.0.56.8 or later |
| --- | --- |
| Document Type: | Configuration & Operations Guide |
| Audience: | Network Administrators,AC Operators |
| Date: | August 20, 2026 |

Revision: 1.0

| Revision | Date | Description |
| --- | --- | --- |
| 1.0 | 2026-08-20 | Initial release |
| 1.1 | 2026-09-30 | Added filtering by Permit MAC and VLAN |

Table Of Content

1. Introduction	4

2. Prerequisites	4

3. Accessing DPSK User Management	4

4. Creating a DPSK User	5

5. Editing DPSK Users	7

6. Modifying SSID Rules	9

7. Deleting DPSK Users	10

8. Batch Import (XLSX)	10

9. Exporting DPSK Users	11

11. Troubleshooting	12

12. Best Practices	12

13. Appendix	13

1. Introduction

1.1 Purpose

This guide provides comprehensive, step-by-step instructions for configuring and managing DPSK (Dynamic Pre-Shared Key) users on the Westala Unify Access Controller (UnifyAC). It is designed to help network administrators efficiently deploy, monitor, and maintain per-user PSK credentials in enterprise wireless environments.

1.2 Scope

This manual applies to the Web UI management of DPSK user life-cycle operations on UnifyAC firmware version UnifyAC_CR_1.1.0.5 and later. It covers user creation, editing, batch operations, SSID/rule binding, access control, expiration policies, and troubleshooting.

1.3 Terminology

| Term | Description |
| --- | --- |
| DPSK | Dynamic Pre-Shared Key — a per-user PSK credential for Wi-Fi authentication |
| AC | Access Controller (UnifyAC) |
| SSID | Service Set Identifier — the Wi-Fi network name |
| VID | VLAN ID — Virtual LAN identifier for traffic segmentation |
| Tx / Rx | Transmit (upload) / Receive (download) bandwidth limits |
| Batch Edit | Simultaneous editing of multiple DPSK users |
| Import / Export | Bulk user management via XLSX file upload/download |

2. Prerequisites

- UnifyAC firmware version UnifyAC_CR_1.0.6.0 or later

- AP firmware version  1.0.58.10 or later

- Supported web browsers: Chrome, Firefox, Edge (latest versions recommended)

- Administrator or Operator role with DPSK configuration privileges

- At least one SSID configured on the UnifyAC prior to DPSK user creation

3. Accessing DPSK User Management

Log in to the UnifyAC Web UI using an administrator account. Navigate to the DPSK User Management page using the following path:

Settings→ WiFi→ DPSK

The DPSK Users interface displays a complete list of existing users, their status, expiration time, bound SSIDs, and DPSK passwords (masked by default). Use the search box to filter by username or Permit Macs and the drop-down filters to filter by SSID, Status or VLAN.

Figure 1 — DPSK Users List View

3.1 Toolbar Functions

The top-right corner of the DPSK Users list contains the following action buttons, ordered from left to right:

| # | Button | Function | Description |
| --- | --- | --- | --- |
| 1 | Batch Edit | Batch Edit | Select multiple users via checkbox, then click to modify common attributes (e.g., Active status, Expire Time) for all selected users simultaneously |
| 2 | Add | Add | Create a new single DPSK user |
| 3 | Import | Import | Open the Batch Import dialog to upload multiple users via an excel template |
| 4 | Export | Export | Export the current DPSK user list to an excel file for backup or offline review |
| 5 | Delete | Delete | Delete selected user(s) |

Tip: Hover over any toolbar icon to display a tool-tip describing its function.

4. Creating a DPSK User

To create a new DPSK user, click the Add (+) button in the toolbar. The User configuration modal will appear with three sections: Basic, SSID & Rule, and Access Control.

Figure 2 — User Creation: Basic Settings

4.1 Basic Settings

| Field | Required | Description |
| --- | --- | --- |
| Username | Yes | Enter a unique identifier for the DPSK user (e.g., user1, john_doe) |
| Password | No | Leave blank for auto-generation, or manually enter a custom password |
| Email | No | Enter the user’s email address |
| Active | Yes | Set to Active to enable the account, or Inactive to disable it |
| Expire Time | Yes | Set the account expiration date and time (Format: YYYY-MM-DD HH:MM:SS) |

4.2 SSID & Rule

In this section, bind the user to specific SSIDs and define network policies:

| Field | Description |
| --- | --- |
| SSID | Select the target SSID from the drop-down list |
| VID | Choose the VLAN ID (e.g., Transparent or a specific numeric VID such as 100) |

✅ If no SSID is specified, the user is rejected on all SSIDs.

Click the + icon to add rules for additional SSIDs. Each SSID can have its own VID.

4.3 Access Control (Optional)

Restrict device access by MAC address for enhanced security:

- Enter a valid MAC address in the Permit MACs field.

- Click + to add it to the allowlist.

- Only devices with listed MAC addresses will be allowed to authenticate using this DPSK.

MAC binding is optional. If no MAC addresses are added, any device may connect using the DPSK credential.

4.4 Saving the User

After completing all required fields, click the Confirm button at the bottom of the modal to save the new DPSK user. Click Cancel to discard changes and close the modal.

5. Editing DPSK Users

5.1 Single User Edit

- Locate the target user in the DPSK Users list.

- Click the Edit (pencil) icon in the Operation column for that user.

- Modify the desired fields in the Edit modal.

- Click Confirm to save changes.

5.2 Batch Editing Users

The Batch Edit feature allows administrators to modify common attributes for multiple users simultaneously, saving significant time in large deployments.

Figure 3 — Batch Edit Mode (select users, then click Batch Edit)

- Check the boxes next to the users you wish to update in the list.

- Click the Batch Edit button (pencil icon) in the top-right toolbar.

- In the Batch Edit modal, update the desired fields (e.g., change Active status or Expire Time).

- Click Confirm. The changes will be applied to all selected users.

6. Modifying SSID Rules

UnifyAC provides two convenient methods for modifying SSID rules for an existing user.

6.1 Inline SSID Rule Edit

To quickly adjust SSID network rules without opening the full edit modal:

Figure 4 — Click the SSID field in the user list to edit rules inline

- Locate the user in the DPSK Users list.

- Click directly on the SSID tag in the SSIDs column (e.g., test1 Transparent).

- In the SSID Rule popup, modify the VID,  or add/remove rules.

- Click Confirm to save the changes.

6.2 SSID Rule Configuration Modal

For more detailed SSID rule management, use the dedicated SSID Rule modal:

Figure 5 — SSID Rule Configuration Modal

| Field | Description |
| --- | --- |
| SSID | The target SSID name |
| VID | VLAN ID drop-down (e.g., wan (100), Transparent) |
| Operation | Delete icon to remove a rule |

Click the + button in the top-right of the modal to add additional SSID rules. Each rule can have independent VID.

7. Deleting DPSK Users

- Select the user(s) you wish to remove by checking the boxes next to their usernames.

- Click the Delete button (trash icon) in the top-right toolbar.

- Confirm the deletion action when prompted.

8. Batch Import (XLSX)

For mass deployment of DPSK users, UnifyAC supports bulk import via XLSX template. This is the most efficient method for on-boarding large numbers of users.

8.1 Import Dialog

Figure 6 — Batch Import Dialog

- Click the Import button (upload icon) in the toolbar.

- In the Batch Import dialog, click Download Template to get the standard XLSX file.

- Fill in the template with the required user data.

- Click Select File to upload the completed XLSX (only .xlsx files are allowed).

- Click Confirm to process the import.

8.2 Template Fields Reference

Figure 7 — XLSX Template with Example Data

| Column | Required | Description |
| --- | --- | --- |
| name | Yes | Username (unique identifier) |
| password | No | DPSK password (leave blank for auto-generation) |
| email | No | User email address |
| user_status | Yes | Active or Deactive |
| ssids | Yes | SSID rules (one per line, see syntax below) |
| expire_time | Yes | Format: YYYY-MM-DD HH:MM:SS |
| permit_macs | No | Comma-separated MAC addresses |

8.3 SSID Rule Syntax

The ssids column in the XLSX template supports the following syntax for defining per-SSID rules:

Format:  ssid_name:vid=N

| Parameter | Description | Example |
| --- | --- | --- |
| ssid_name | Name of the SSID | test1, ssid2 |
| vid=N | VLAN ID (numeric) | vid=10, vid=100 |

Example SSID entries (each on a separate line within the cell):

ssid1:vid=10

ssid2

In the XLSX cell, each SSID rule must be on a separate line (use Alt+Enter in Excel to create a new line within a cell).

9. Exporting DPSK Users

Click the Export button (download icon) in the toolbar to export the current DPSK user list to an XLSX file. This is useful for:

- Creating backups of the current user database

- Offline review and auditing of active accounts

- Migrating users to another AC (export, then import)

11. Troubleshooting

| Issue | Possible Cause | Resolution |
| --- | --- | --- |
| User cannot connect | Account expired or status is Inactive | Verify the Expire Time and Active status in the user list. Update if necessary. |
| DPSK rejected | Incorrect password entered on the client device | Click the eye icon to reveal the correct password and re-enter it on the client. |
| MAC binding failure | Client MAC does not match the Permit MACs list | Ensure the device’s MAC address is correctly added to the user’s Access Control list. |
| Cannot access specific SSID | SSID Rule restricts the user to different SSIDs | Edit the user’s SSID & Rule section to ensure the target SSID is included. |
| Import file fails | Incorrect file format or syntax error in XLSX | Ensure the file is .xlsx and the ssids column follows the exact required syntax. |
| User connects but has no internet | VLAN or VID misconfiguration | Verify the VID setting in the SSID rule matches the network’s VLAN configuration. |

12. Best Practices

- Set Expiration: Always define an Expire Time for temporary guests or contractors to ensure automatic deactivation.

- VLAN Segmentation: Use specific VIDs in SSID rules to segment user traffic for security and policy enforcement.

- MAC Binding: Enable MAC-based access control for high-security environments (e.g., corporate executives, finance departments).

- Regular Audits: Periodically export the DPSK list to review active users and remove obsolete accounts.

- Bulk Operations: Use Batch Import for initial deployment and Batch Edit for mass updates to save administrative time.

13. Appendix

13.1 Example User Entry

| Field | Value |
| --- | --- |
| Username | user1 |
| Password | Auto-generated |
| Email | user1@email.com |
| Status | Active |
| Expire Time | 2036-08-19 18:32:08 |
| SSIDs | test1 (VID 100) |
| MAC Control | 11:22:33:44:55:66 |

13.2 Support

For additional assistance, please contact Westala Technical Support or refer to the UnifyAC Administrator Reference Manual.